Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#01)

INTRODUCTION TO VERILOG

Student Name:  M SHAFAY UL HASSAN_ Roll No: 24I-6552 Section: CE-A__

Date performed: 18-08, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

Implemented the 7 basic gates with the Gate Level approach. Gate Level approach is that you have to implement the gates with the gates keyword like and,or,not etc.

Code:

module AND_gate(a,b,c); //AND Gate input a; input b; output c; and chk(c,a,b); endmodule module AND_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; AND_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule // OR Gate module OR_gate(a,b,c); //or Gate input a; input b; output c; or chk(c,a,b); endmodule module OR_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; OR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //not gate module NOT_gate(a,c); //AND Gate input a; output c; not chk(c,a); endmodule module NOT_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; wire z; NOT_gate uu(.a(x),.c(z)); initial begin x=0; #50 x=1; end endmodule //NoR Gate module NOR_gate(a,b,c); input a; input b; output c; nor chk(c,a,b); endmodule module NOR_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; NOR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //XOR Gate module XOR_gate(a,b,c); input a; input b; output c; xor chk(c,a,b); endmodule module XOR_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; XOR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //NAND Gate module NAND_gate(a,b,c); input a; input b; output c; nand chk(c,a,b); endmodule module NAND_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; NAND_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule // XNOR Gate module XNOR_gate(a,b,c); input a; input b; output c; xnor chk(c,a,b); endmodule module XNOR_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; XNOR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule

Output:

AND Gate: OR Gate: NOT Gate: NOR Gate: XNOR Gate: NAND Gate: XOR Gate:

Task 2:

Description:

Implemented the 7 basic gates with the Data Flow Level Approach Data Flow Level approach is that you have to implement the gates with the assign keyword like assign c=a&b.

Code:

module data_OR_gate(a,b,c); //or Gate input a; input b; output c; assign c=a¦b; endmodule module data_OR_test_bench(); // the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; data_OR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //and Gate module data_AND_gate(a,b,c); input a; input b; output c; assign c=a&b; endmodule module data_AND_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; data_AND_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //not Gate module data_NOT_gate(a,c); input a; output c; assign c=~a; endmodule module data_NOT_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; wire z; data_NOT_gate uu(.a(x),.c(z)); initial begin x=0; #50 x=1; #50; end endmodule //NOR Gate module data_NOR_gate(a,b,c); input a; input b; output c; assign c=~(a¦b); endmodule module data_NOR_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; data_NOR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //XOR Gate module data_XOR_gate(a,b,c); input a; input b; output c; assign c=a^b; endmodule module data_XOR_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; data_XOR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //Nand Gate module data_NAND_gate(a,b,c); input a; input b; output c; assign c=~(a&b); endmodule module data_NAND_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; data_NAND_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule //XNOR Gate module data_XNOR_gate(a,b,c); input a; input b; output c; assign c=~(a^b); endmodule module data_XNOR_test_bench();// the input vars would be of reg type and the output vars would be of wire type reg x; reg y; wire z; data_XNOR_gate uu(.a(x),.b(y),.c(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; end endmodule

Output:

AND Gate: OR Gate: NOT Gate: NOR Gate: XNOR Gate: NAND Gate: XOR Gate:

Task 3:

Description:

Implemented the gate level and data flow level of Verilog codes of the first task in logeism to verify the results in both the logism and modelsim . This technique is very useful to verify your results.

Output:

Task 4:

Description:

Implemented the 7 basic gates with the Data Flow Level Approach Data Flow Level approach is that you have to implement the gates with the assign keyword like assign c=a&b.Solved the complex circuit by using wires and output and input keywords .

Code:

module comp_big(a,b,out); input a; input b; output out; wire t1; wire t2; wire ta; wire tb; not temp1(t1,a); and temp2(ta,t1,b); not temp3(t2,b); and temp4(tb,a,t2); or tt(out,ta,tb); endmodule module test_bench04(); reg x; reg y; wire z; comp_big tt(.a(x),.b(y),.out(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; #50; end endmodule

Output:

Task 5:

Description:

Implemented the gate level and data flow level of Verilog codes of the first task in logeism to verify the results in both the logism and modelsim . This technique is very useful to verify your results .

Output:

Task 6:

Description:

Implemented the gate level approach to solve the one`s compliment problem by taking the 4 inputs and 4 outputs and check the result using the waveforms .Also used wires in the test bench for the simulation results.

Code:

module comp_big(a,b,out); input a; input b; output out; wire t1; wire t2; wire ta; wire tb; not temp1(t1,a); and temp2(ta,t1,b); not temp3(t2,b); and temp4(tb,a,t2); or tt(out,ta,tb); endmodule module test_bench04(); reg x; reg y; wire z; comp_big tt(.a(x),.b(y),.out(z)); initial begin x=0;y=0; #50 x=1;y=0; #50 x=0;y=1; #50 x=1;y=1; #50; end endmodule

Output: